ARDUINO REMOTE LAB (2nd practice)

- Write a program that shows messages when a button or switch is pressed.

  - If the user presses the button 1, a message like this must appear: Button 1 pressed

  - Do the same with the other buttons

Recommendation: don’t forget to use time blocks. For example, wait one second every time you detect the button or switch.

- Write a program that controls one LED using a button.

  - If the user presses the button 1 (digital input 13), for example, one LED will turn on (digital output 8). If the button isn’t pressed, the LED will be off

  - Check the same program with the switch and see the differences between the button and the switch

- Write a program so:

  - If the user presses the button 1 (digital 13), one LED (digital 8) must be ON and the others OFF during two seconds.

  - If the user presses the button 2 (digital 12), two LEDs (digital 8 and 7) must be ON and the others OFF during two seconds.

  - If the user presses the button 3 (digital 4), three LEDs (digital 8, 7 and 6) must be ON and the other OFF during two seconds.

  - If the user presses the button 1 (digital 2), all the LEDs (digital 8, 7, 6 and 5) must be ON.

  - If none is pressed all the LEDs must be off.

  - The program must print (using comms block and the block that is in the picture) which LEDs are ON. For example:

    - LED 1 is ON

    - LED 1 and 2 are ON

    - …

- Make this program:

  - Each time the button (for example the button 1) is pressed we need to turn on an LED in this practice (for example, the 8)

  - Also every time we press that button, we will add value to a counter and show it using serial comms

  - When the counter reaches 9, you will turn on the RED LED and set it to 0 again to start counting again.

- Write a program that:

  - Moves the motor from 0 to 180, in steps of 5º.

  - It will be one second still in 180

  - Moves the motor from 180 to 0, in steps of 5º

  - It will be one second still in 180

- Write a program that:

  - If the button 1 is pressed, the motor must move from 0 to 180 and then from 180 to 0, and the RED LED must be on during this process.

  - If the button 2 is pressed, the motor must move from 0 to 90 and then from 90 to 0, and the GREEN LED must be on during this process

HOW DO I SEND THE EXERCISES?

1- When you are sure that your exercise is correct, click on “Export” in the menu of the next picture.

2- After clicking on the button, the file (with xml extension) will be downloaded to Drive

3- Rename the file with the title of the practice, the exercise and the names of the members. For example: Pr1Ex1_DavidAlazneJulen

4- Upload to Classroom